Worksheet 1 – Comparing Bulb types

| Student name |  | Date |  |
| --- | --- | --- | --- |
| Topic: | Basic level - What is inside a light bulb? 1 Medium level - Efficiency and lifetime of light bulbs 5 Advanced level - Total cost of ownership and environmental impact 9 |  |  |
| Learning outcomes | Basic level: - Students will recognise the four main types of household light bulbs. - Students will identify the wattage and lifetime of common bulb types. Medium level: - Students will compare the four main types of light bulbs on energy use, lifetime and cost. - Students will calculate the luminous efficacy (lumens per watt) of each type. - Students will reason about the trade-off between purchase price and running cost. Advanced level: - Students will model the total cost of ownership (TCO) of different bulb types as a function of usage hours. - Students will perform a sensitivity analysis on electricity tariff and usage intensity. - Students will quantify lifecycle CO2 emissions and discuss the embodied carbon of bulb manufacturing. |  |  |
| Time needed | 60 – 90 – 120 min |  |  |
| Resources | Basic level: Bulb information table (in this worksheet); access to one or more real bulbs if available. Medium level: Bulb specification table (in this worksheet); calculator. Advanced level: Bulb specification table; calculator or spreadsheet software (recommended); access to current Dutch electricity tariff data. |  |  |

# Basic level - What is inside a light bulb?

Introduction

Not all light bulbs are the same. Some get hot, some last for years, and some use much less electricity. In this worksheet you will look at four common types of bulbs and learn the basic differences between them.

Theory

Every light bulb does the same job: it turns electricity into light. But not all bulbs do that equally well. Some bulbs waste a lot of electricity by also turning it into heat. That is why an old-fashioned bulb feels hot after only a minute, while an LED stays cool.

There are four bulb types you will see most often at home and at school. They all use the same electrical socket but they work in completely different ways inside.

INCANDESCENT bulbs (the oldest type) have a thin metal wire inside. When electricity flows through the wire it gets so hot that it glows white. Most of the energy actually becomes heat, not light. These bulbs are no longer sold in most EU countries.

HALOGEN bulbs are an improved version of the incandescent bulb. They contain a small amount of halogen gas which makes the wire last a bit longer and burn slightly brighter. Still mostly heat though.

COMPACT FLUORESCENT (CFL) bulbs use a folded glass tube filled with gas. Electricity makes the gas glow. They use much less electricity than incandescent bulbs but contain a tiny amount of mercury, so they need to be recycled carefully.

LED bulbs (Light Emitting Diode) use a small electronic component that produces light directly when electricity flows through it. They are the most efficient: they convert almost all the electricity into light, and very little into heat. They also last 25 times longer than an incandescent bulb.

Worked example How do you read the bulb information table? Step 1: Look at the row 'Lifetime'. The number tells you how many HOURS the bulb works before it needs to be replaced. Step 2: Example: an LED bulb has a lifetime of 25,000 hours. If you use it 5 hours per day, that is 25,000 ÷ 5 = 5,000 days, which is about 13 years. Step 3: Look at the row 'Wattage'. This tells you how much ELECTRICITY the bulb uses each second it is on. A higher wattage means more electricity. Step 4: Example: an incandescent bulb of 60 W uses 5 times more electricity than an LED of 12 W to give the same amount of light.

Key terms

| Term | Definition |
| --- | --- |
| Bulb | A light source that screws or clicks into a lamp. |
| Lifetime | The number of hours a bulb works before it stops working. |
| Wattage (W) | How much electricity the bulb uses. |
| Lumens (lm) | How much light the bulb produces (more lumens = brighter). |
| LED | Light Emitting Diode - the most efficient bulb type today. |

Student activity 1

Recognising the four bulb types

Look at the four bulb types in the table below.

|  | Incandescent bulb | Halogen | Compact fluorescent | Light emitting diode (LED) |
| --- | --- | --- | --- | --- |
| Lifetime | 1,000 h | 3,000 h | 10,000 h | 25,000 h |
| Price per bulb | €0.50 | €1.50 | €1.50 | €1.33 |
| Wattage | 60 W | 43 W | 13 W | 12 W |

- Which bulb has the longest lifetime?

- Which bulb uses the most electricity (highest wattage)?

- Which bulb uses the least electricity (lowest wattage)?

- Which bulb is the cheapest to buy?

- All four bulbs give about the same amount of light (850 lumens). Which bulb would you choose for a room you use every day? Explain in one sentence why.

Student activity 2

Match the bulb to the description

Draw a line (or write the matching letter) connecting each bulb type to the correct description.

- Incandescent bulb → ___ | A. Uses a coiled tube filled with gas; was popular before LEDs

- Halogen bulb → ___ | B. The newest technology; very low energy use and long life

- Compact fluorescent → ___ | C. The oldest technology; a thin wire glows when electricity passes through

- LED → ___ | D. An improved version of the incandescent; slightly more efficient

# Medium level - Efficiency and lifetime of light bulbs

Introduction

Light bulbs differ in three important ways: how much electricity they use, how long they last, and how much they cost to buy. Modern LEDs combine low energy use with a long lifetime, but they are still more expensive to buy than older bulb types. In this worksheet you will quantify these trade-offs.

What you should already know: You should already know the four common bulb types and what wattage and lifetime mean (see Worksheet 1A if needed).

Theory

All four bulb types in the table give about the same amount of light: 850 lumens. So the only meaningful comparison between them is: how much electricity, money and replacement effort does each one cost to produce that light?

LUMINOUS EFFICACY is the standard way to express how 'good' a light source is at converting electricity into light. It is measured in lumens per watt (lm/W). The formula is:

Efficacy (lm/W) = Light output (lumens) / Power (watts)

Modern LEDs reach 70 to 100 lm/W. An old incandescent bulb only reaches about 14 lm/W. So an LED is roughly 5 to 7 times more efficient.

TOTAL COST OF OWNERSHIP (TCO) is the second key concept. The price you pay in the shop is only part of the cost. You also pay for the electricity the bulb uses and for replacement bulbs when one burns out. To compare two bulb types fairly, you must add all these costs over the same number of hours.

Worked example Calculate the luminous efficacy of a halogen bulb. Step 1: Read from the table: Halogen produces 850 lumens at 43 W. Step 2: Apply the formula: Efficacy = 850 / 43 ≈ 19.8 lm/W. Step 3: Compare: this is about 1.4 times better than incandescent (850 / 60 ≈ 14.2 lm/W) but about 4 times worse than LED (850 / 12 ≈ 70.8 lm/W).

Key terms

| Term | Definition |
| --- | --- |
| Luminous efficacy | Lumens of light produced per watt of electricity (lm/W). Higher = better. |
| Total cost of ownership (TCO) | Purchase cost + electricity cost + replacement cost over the full usage period. |
| Tariff (τ) | The price of electricity in € per kWh. |
| kWh | Kilowatt-hour. The unit your electricity company uses for billing. 1 kWh = 1000 W used for 1 hour. |

Student activity 1

Luminous efficacy

All four bulb types in the table provide about 850 lumens of light. The luminous efficacy of a bulb tells you how many lumens you get per watt of electricity:

Efficacy (lm/W) = Light output (lumens) / Power (watts)

|  | Incandescent bulb | Halogen | Compact fluorescent | Light emitting diode (LED) |
| --- | --- | --- | --- | --- |
| Lifetime | 1,000 h | 3,000 h | 10,000 h | 25,000 h |
| Price per bulb | €0.50 | €1.50 | €1.50 | €1.33 |
| Wattage | 60 W | 43 W | 13 W | 12 W |
| Light output | ≈850 lm | ≈850 lm | ≈850 lm | ≈850 lm |

- Calculate the luminous efficacy for each of the four bulb types.

- By what factor is the LED more efficient than the incandescent bulb?

- What does this mean for energy consumption when both bulbs are used for the same number of hours?

Student activity 2

Cost over the lifetime of the bulbs

Assume you want light for 10,000 hours (about 4 years if used 7 hours per day).

- How many bulbs of each type do you need to reach 10,000 hours of light? Round up to whole bulbs.

- What is the total purchase cost for each bulb type to reach 10,000 hours?

- Use Energy (kWh) = Power (kW) × time (h) to calculate the energy used in 10,000 hours for each type.

- Calculate the electricity cost for each type. Use a tariff of €0.30/kWh.

- Add purchase cost and electricity cost. Which bulb type is the cheapest in total?

# Advanced level - Total cost of ownership and environmental impact

Introduction

When choosing between light bulbs, the cheapest option depends on how long you use them. For a lamp that is on for only a few minutes per day, an LED may never pay back its higher purchase price. For a lamp that burns 12 hours a day, an LED is far cheaper within a year. In this worksheet you will build a Total Cost of Ownership (TCO) model and analyse when each technology is the rational choice.

What you should already know: This worksheet assumes you have completed Worksheet 1B (or are confident with luminous efficacy, kWh calculations, and the simple TCO concept).

Theory

At this level we treat bulb selection as an optimisation problem. The TCO function from worksheet 1B can be written more rigorously and analysed mathematically.

The continuous form of TCO over a usage period of t hours is:

TCO(t) ≈ (t / L) · P_bulb + (W · t / 1000) · τ

where L is bulb lifetime (h), P_bulb is the unit purchase price (€), W is the wattage and τ is the electricity tariff (€/kWh). Strictly speaking, you cannot buy 0.7 of a bulb, so the realistic version uses the ceiling function ⌈t/L⌉ for the number of bulbs needed.

BREAK-EVEN ANALYSIS: comparing two bulb types A and B, the break-even hours t* is the value of t where TCO_A(t) = TCO_B(t). Below t*, the cheaper-to-buy option wins; above t*, the more efficient option wins. Solving for t* algebraically gives insight into when switching technology actually pays off.

SENSITIVITY: real-world conclusions depend on input assumptions (especially τ, which has varied between €0.10 and €0.70 per kWh in the Netherlands in recent years). A robust analysis tests how the conclusion changes when key inputs change.

LIFECYCLE CO₂: the total environmental impact includes both manufacturing emissions (embodied carbon) and emissions from electricity used during the bulb's lifetime. Using the 2024 Dutch grid factor of 0.27 kg CO₂/kWh, manufacturing typically dominates LED lifecycle emissions whereas electricity use dominates incandescent lifecycle emissions.

Worked example Find the break-even hours between an incandescent bulb (W=60, P=€0.50, L=1,000 h) and an LED (W=12, P=€1.33, L=25,000 h) at τ=€0.30/kWh. Step 1: Use the continuous TCO: TCO_inc(t) = (t/1000)·0.50 + (60t/1000)·0.30 = 0.0005·t + 0.018·t = 0.0185·t Step 2: TCO_LED(t) = (t/25000)·1.33 + (12t/1000)·0.30 = 0.0000532·t + 0.0036·t ≈ 0.003653·t Step 3: Set equal: 0.0185·t = 0.003653·t → this gives 0 = -0.014847·t, so they are equal only at t = 0. Step 4: Conclusion: with these prices the LED is cheaper from hour 1 onwards (its TCO grows much more slowly). The 'break-even' question only becomes interesting if the LED's purchase price is much higher (e.g. €15), in which case there is a real positive break-even point.

Student activity 1

Building a TCO model

The total cost of ownership over a period of t hours can be written as:

TCO(t) = ⌈t / L⌉ × P_bulb + (W × t / 1000) × τ

where L is the lifetime of one bulb (h), P_bulb is the purchase price (€), W is the wattage (W), τ is the electricity tariff (€/kWh), and ⌈·⌉ is the ceiling function (round up to whole bulbs).

|  | Incandescent bulb | Halogen | Compact fluorescent | Light emitting diode (LED) |
| --- | --- | --- | --- | --- |
| Lifetime | 1,000 h | 3,000 h | 10,000 h | 25,000 h |
| Price per bulb | €0.50 | €1.50 | €1.50 | €1.33 |
| Wattage | 60 W | 43 W | 13 W | 12 W |
| Light output | ≈850 lm | ≈850 lm | ≈850 lm | ≈850 lm |

- Derive a simplified version of this formula by assuming continuous bulb replacement (i.e. ignore the ceiling function). Show your derivation.

- Plot (or sketch) TCO(t) for all four bulb types on the same axes for t between 0 and 25,000 hours. Use τ = €0.30/kWh.

- Calculate the break-even point in hours between (a) incandescent vs. LED and (b) CFL vs. LED. Below which usage does the older technology still 'win' on TCO?

Student activity 2

Sensitivity analysis

Real-world electricity tariffs in the Netherlands have varied between €0.10/kWh and €0.70/kWh in the last five years.

- Recompute the LED-vs-incandescent break-even point for τ = €0.10 and τ = €0.70. How sensitive is the conclusion to electricity price?

- A user only uses a particular lamp 30 minutes per day. Over the 25,000-hour LED lifetime, how many calendar years is that? Is the LED still rational here?

Student activity 3

Lifecycle CO₂

1 kWh of Dutch grid electricity emits approximately 0.27 kg of CO₂ (2024 average; was 0.46 kg in 2018). Manufacturing one LED bulb is estimated to emit ~1.5 kg CO₂; one incandescent bulb ~0.3 kg.

- Calculate the total lifecycle CO₂ emissions (manufacturing + use) for 25,000 hours of light, for both LED and incandescent. Use the 2024 grid factor.

- What share of total LED lifecycle emissions comes from manufacturing? What does this imply about the value of replacing working incandescent bulbs versus waiting until they fail?