Worksheet

3.1. Cleaning & Household Maintenance: Washing machines and dryers

Level: Basic/intermediate

| Student name |  | Date |  |
| --- | --- | --- | --- |
| Topic: | Washing machines and dryers |  |  |
| Learning outcomes | - Technical knowledge: Students learn how washing machines and dryers work, consume energy/water, and the difference between vented and condenser dryers. - Mathematical skills: Students calculate the cost savings of using eco/wash programmes and compare dryer types. - Environmental awareness: Students understand how laundry habits affect carbon footprint and water scarcity. - Green skills: Students evaluate trade-offs between convenience, cost, and environmental impact. - Students will compare energy and water consumption between standard and eco settings. - Students will evaluate the efficiency of vented vs. condenser dryers. |  |  |
| Time needed | 90 min |  |  |
| Resources | Calculator, electricity tariff (e.g., 0.30 €/kWh), water tariff (e.g., 0.002 €/l), appliance datasheets. |  |  |

Introduction

- The average European household does 3–5 wash loads per week. In this worksheet you will investigate how much energy and water a washing machine uses on different programmes, compare types of tumble dryer, and calculate what you could save by choosing eco settings.

Theory

- A washing machine is one of the biggest water and electricity users in a home. Most of the energy goes into heating water – not spinning the drum. Modern washing machines include eco modes that use less water and lower temperatures, though they often take longer to run.

Heating water to 40°C or 60°C uses far more energy than running the motor. An Eco program at 20°C – 30°C uses up to 60% less energy but runs longer.

The trade-off: longer cycle time but lower cost and less fabric damage.

- Clothe dryers are even more energy intensive. Understanding the difference between vented and condenser dryers – and when to avoid using a dryer at all – can save money and reduce emissions.

- Dryer types:

- Vented dryer: Pulls in room air, heats it, blows humid air out through a hose (must be vented outside). High energy use (typical: 2.5–3.5 kWh per load).

- Condenser dryer: No vent needed. Moisture is collected in a tank. Slightly more efficient (2.0–3.0 kWh per load) but slower.

- Heat pump dryer: Most efficient (1.0–1.5 kWh per load) – uses a refrigerant cycle like a fridge. It is more expensive to buy but cheaper to run.

- Water use: Washing machines typically use 40–90 litres per cycle, depending on drum size and programme. Eco programmes tend to use less water because they spin more slowly and re-use rinse water more efficiently.

- Key formula for water heating:
Energy to heat water = liters×4.18×ΔT/3600 kWh (simplified: ~0.05 kWh per litre per 10°C temperature rise).

- Watch videos to get an intro about how washing machines work and what is the difference between quick cycles and eco cycles:

- How Do Washing Machines Clean Our Clothes?

- Washing Machine Quick Washes v's Eco Washes

Worked example - A cotton 60 °C programme uses 1.8 kWh and 60 l per wash. The eco 40 °C programme uses 0.9 kWh and 45 l. The household does 4 washes per week. - Annual energy difference: (1.8 – 0.9) × 4 × 52 = 187.2 kWh - Annual cost saving at 0.30 €/kWh: 187.2 × 0.30 = 56.16 € per year - Annual water saving: (60 − 45) × 4 × 52 = 3,120 l

Student activity 1

Washing machine – cold vs. hot

An average 4-person household washes 220 loads per year.

| Programme | Water temp | Energy per cycle (kWh) | Water per cycle (L) | Programme |
| --- | --- | --- | --- | --- |
| Cotton 60°C | 60°C | 1.8 | 55 | Cotton 60°C |
| Eco 40°C | 40°C | 0.9 | 45 | Eco 40°C |
| Cold 20°C | 20°C | 0.3 | 45 | Cold 20°C |

- Electricity tariff: 0.30 €/kWh.

- Water tariff: 0.002 €/l

- Calculate the annual electricity cost for each programme.

- Calculate the annual water cost for each programme.

- What is the total annual saving switching from Cotton 60°C to Eco 40°C?

- How many kg of CO₂ are saved per year? (Use 0.27 kg CO₂ per kWh)

- Find an EU energy label for a washing machine online (rating A to G). What is the difference in kWh/year between an A-rated machine and a D-rated machine?

Watch videos to get an intro about how clothes dryers work:

- The 3 Types of Tumble Dryer Explained (Vented, Condenser, Heat Pump)

- How Does a Heat Pump Tumble Dryer Work?

Student activity 2

Dryer comparison

A household dries 180 loads of laundry per year.

| Dryer type | Energy per load (kWh) | Purchase price (€) | Lifetime (years) |
| --- | --- | --- | --- |
| Vented | 3.2 | 350 | 8 |
| Condenser | 2.8 | 450 | 10 |
| Heat pump | 1.2 | 800 | 12 |

Electricity cost: 0.30 €/kWh.

- Calculate the annual running cost for each dryer type.

- Calculate the 10-year total cost (purchase + running) for each.

- Which dryer is cheapest overall after 10 years?

- If the CO₂ emission factor for electricity is 0.27 kg CO₂/kWh:
a) Calculate the annual CO₂ emissions for each dryer type (vented, condenser, heat pump) with 180 cycles per year.
b) How many kg CO₂ per year does a heat pump dryer save compared to a vented dryer?
c) How many trees absorb that amount of CO₂ annually (1 tree ~22 kg CO₂/year)?

Did you know?

A clothesline or drying rack uses 0 kWh.

Peer-to-peer tip

- If the household air-dries 50% of loads instead of using a vented dryer, how much money do they save per year?

Reflection

- Why do Eco programmes take longer? Is that a problem in practice?

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

- When would a condenser dryer be preferred over a heat pump dryer despite higher running costs? (Think about space, installation, upfront budget.)

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

- If every household in a city of 100,000 switched to eco washing programme (saving 93.6 kWh/year), how many kWh would be saved city-wide? Convert to MWh and GWh.

Additional tasks for electrical engineering students

- Draw a block diagram of a washing machine (heater, motor, pump, control board, sensors), indicate typical power ratings of each component.

- What type of motor is used in modern washing machines (universal motor, BLDC)? What are the advantages of BLDC?

- Are household washing machines connected to single-phase or three-phase power? Explain why.

- What protection must a washing machine have (thermal, overcurrent, leak protection)?

- Explain how an NTC thermistor regulates water temperature in a washing machine.

- A washing machine motor has a power factor of 0.75. The apparent power (VA) is higher than real power (W). Calculate the apparent power for a 500 W motor. Why does this matter for the electricity grid?

- Draw a block diagram of a dryer on the type you choose (heater, motor, pump, control board, sensors), indicate typical power ratings of each component.