Worksheet

3.2. Energy and water efficiency of dishwashers

Level: Basic/intermediate

| Student name |  | Date |  |
| --- | --- | --- | --- |
| Topic: | Energy and water efficiency of dishwashers |  |  |
| Learning outcomes | - Technical knowledge: Students can explain how dishwashers consume energy (heating water) and water per cycle. - Mathematical skills: Students calculate annual costs using realistic usage scenarios. - Environmental awareness: Students compare energy and water consumption across dishwasher programmes. - Green skills: - Students read energy labels and choose appliance settings. - Students compare annual costs of different programmes and estimate savings from eco settings. - Students evaluate the environmental |  |  |
| Time needed | 60 min |  |  |
| Resources | Calculator, electricity tariff (e.g., 0.30 €/kWh), water tariff (e.g., 0.002 €/l), EU energy label for a dishwasher (from home or manufacturer website) |  |  |

Introduction

- A modern dishwasher can use less water than washing by hand – but only if it is loaded correctly and run on the right programme. The choice between an intensive 70 °C programme and an eco-programme can make a significant difference to your electricity bill and water use. In this worksheet you will investigate those differences and decide when each programme makes sense.

Theory

Key formula for water heating:
Energy to heat water = liters×4.18×ΔT/3600 kWh (simplified: ~0.05 kWh per litre per 10°C temperature rise).

Did you know?

A clothesline or drying rack uses 0 kWh

A dishwasher uses most of its energy to heat water. It also uses a small amount for the drying cycle (heating element).

- Intensive programme (70 °C): Uses more water (18–20 l) and more electricity (1.5–2.0 kWh) – meant for pots and heavily soiled dishes.

- Auto programme: Sensors adjust temperature and duration – convenient but often less efficient than Eco.

- Eco programme (50 °C): Long cycle (3–4 hours) but lower temperature. Typical: 0.8–1.0 kWh, 10–12 l of water.

- Hand-washing comparison: A running tap uses 6–12 l of water per minute. Washing dishes for 10 minutes with the tap half-open = 60–120 l – far more than a dishwasher.

- Eco label: Since 2021, dishwashers have carried an EU energy label rating from A (best) to G. The label shows: energy per cycle (kWh), water per cycle (liters), noise level (dB), and programme duration.

- Half-load: Running a half-loaded machine wastes energy and water. If you have a half-load setting, it typically saves 20–30% on both. Otherwise, waiting for a full load is more efficient.

- Watch videos to get an intro about how dishwashers work and what is the difference between quick cycles and eco cycles:

- How Does a Dishwasher Actually Work?

- Dishwasher Eco Setting Explained

Worked example - A household runs the dishwasher 250 times per year. - Intensive programme: 1.9 kWh, 19 l water. Cost per cycle = (1.9 × 0.30 €) + (19 × 0.002 €) = 0.57 € + 0.038 € = 0.608 €. - Eco programme: 0.9 kWh, 11 l water. Cost per cycle = (0.9 × 0.30 €) + (11 × 0.002 €) = 0.27 € + 0.022 €= 0.292 €. - Savings per cycle: 0.316 €. - Annual saving: 250 × 0.316 € = 79 €.

Student activity 1

Compare dishwasher programmes

| Programme | Water temp | Energy per cycle (kWh) | Water per cycle (l) |
| --- | --- | --- | --- |
| Intensive (70°C) | 1.8 | 20 | 90 |
| Auto | 1.2 | 15 | 120 |
| Eco (50°C) | 0.85 | 11 | 210 |
| Rinse only (no heat) | 0.05 | 4 | 15 |
| Your machine's eco |  |  |  |

- Electricity: €0.30/kWh. Water: €0.003/L.

- How many times does your household run the dishwasher per year? _____ cycles/year.

- Calculate the annual cost for each programme at your usage frequency.

- How much money is saved per year by using Eco instead of Intensive?

- How much water is saved per year (in liters) at your usage frequency?

- Why does the Eco programme take over 3 hours? (Hint: Think about heating rate and insulation.)

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

Student activity 2

Dishwasher vs. hand-washing

- A person hand-washes dishes for 12 minutes with the tap at 8 l/min. The water is heated by an electric boiler (0.05 kWh/l to heat from 10°C to 40°C).

- Calculate total water used for hand-washing.

- Calculate energy to heat that water (kWh).

- Calculate total cost for hand-washing (water + electricity).

- Compare to the Eco programme cost from Activity 1. Which is cheaper?

______________________________________________________________________

______________________________________________________________________

- For what types of washing loads is the intensive programme justified? Give two real examples.

______________________________________________________________________

- If the school canteen runs 10 dishwasher cycles per school day (160 days/year) on the intensive programme, calculate the annual energy cost. Then recalculate if they switch to eco. What is the annual saving?

- Find an EU energy label for a dishwasher. What is the difference in annual energy (kWh/year) between A and E ratings?

Student activity 3

Carbon and water footprint

- Calculate the CO₂ emissions from 365 intensive cycles (use 0.27 kg CO₂/kWh).

- How does switching to eco reduce the annual carbon footprint in kg CO₂?

- The water cost is 0.003 €/l for water. Re-calculate the annual water saving in euros using this tariff. Does this change the economic argument for eco?

Did you know?

Scrape don't rinse!  – modern dishwashers don't need pre-rinsing.

Peer-to-peer tip

- How does pre-rinsing waste water and energy?

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

Reflection

- When would hand-washing be more efficient than a dishwasher?

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

Additional tasks for electrical engineering students

- Draw a block diagram of a dishwasher showing: control board, heating element, circulation pump, drain pump, NTC thermistor, door interlock switch, detergent dispenser, flood protection sensor. Indicate typical power ratings.

- Modern dishwashers use BLDC (brushless DC) motors for the circulation pump. List three advantages of BLDC over universal motors. Why are they more expensive?

- How is the water temperature regulated? Explain.

- List three safety devices in a dishwasher.

- An NTC thermistor has a resistance of 10 kΩ at 25°C and 2 kΩ at 60°C. If it is used in a voltage divider circuit with a 5V supply and a 10 kΩ fixed resistor, calculate the output voltage at 25°C and at 60°C. How does the control board use this signal?