Worksheet 2B

I understand the energy classes?

Level: MEDIUM

| Student name |  | Date |  |
| --- | --- | --- | --- |
| Topic: | Energy Ratings – Understanding Energy Classes |  |  |
| Learning outcomes | - Interpret EU energy labels (A–G) - Compare device efficiency - Calculate energy consumption and cost |  |  |
| Time needed | 45 – 60 min |  |  |
| Resources | - Device energy label (monitor/TV) - Calculator |  |  |

Introduction

When buying a television, monitor or other electronic device, consumers often focus on price and features. However, energy efficiency is equally important. Energy labels help consumers compare products and choose devices that use less electricity while providing the same service.

Choosing efficient products reduces electricity bills and contributes to protecting the environment.

Theory

The European Union uses a standard energy label system ranging from Class A (most efficient) to Class G (least efficient).

The label also provides information about annual energy consumption. Efficient devices require less electricity to perform the same task, reducing both operating costs and carbon emissions.

A more efficient television may cost slightly more when purchased, but it often saves money throughout its lifetime through lower electricity consumption.

Did you know?

Even an 'A' rated monitor can waste energy if the brightness is at 100% all the time. Lowering brightness to a comfortable 60% can reduce power consumption by nearly 20% without hurting your eyes!

Worked example Television A: 50 kWh/1000 hours Television E: 100 kWh/1000 hours If both are used 5 hours per day: Annual hours: 5 × 365 = 1825 hours TV A: 50 × 1825 ÷ 1000 = 91.25 kWh/year TV E: 100 × 1825 ÷ 1000 = 182.5 kWh/year The less efficient television uses twice as much energy.

Key terms

| Term | Definition |
| --- | --- |
| Energy Label | EU system for comparing product efficiency |
| Energy Class | Rating from A to G |
| Efficiency | Producing the same result with less energy |
| Consumption | Amount of energy used |
| Carbon Footprint | Greenhouse gases produced by human activities |

Student activity 1

Task 1 – Label Analysis

Fill in:

Energy class: ______

Energy consumption: ______ kWh/1000h

Technology: ______

Task 2 – Usage Calculation

Daily use = 5 hours

Yearly hours:

TYH = 5 × 365 = ______ hours

Yearly energy:

YEC = ______ /1000 × ______ = ______ kWh

Task 3 – Cost Comparison

Electricity price (Portugal): ______ €/kWh

Annual cost:

TAC = ______ × ______ = ______ €

Task 4 – Compare Two Devices

| Device | Class | kWh/year | Cost/year |
| --- | --- | --- | --- |
| A-rated |  |  |  |
| E-rated |  |  |  |

Savings per year: ______ €

Task 5 – Environmental Impact

CO₂ factor ≈ 0.4 kg/kWh

CO₂ emissions:

______ × 0.4 = ______ kg CO₂/year