Worksheet 2 – Cooking appliances

| Student name |  | Date |  |
| --- | --- | --- | --- |
| Topic: | Basic level – Which cooking method uses the most electricity? Medium level – Comparing cooking appliances: energy, time and cost Advanced level – Optimising kitchen energy use and carbon footprint |  |  |
| Learning outcomes | Basic level: Students will identify common cooking appliances, understand their power ratings, and recognise which cooking methods use the most energy. Medium level: Students will calculate energy consumption for different cooking methods, compare costs, and evaluate efficiency improvements. Advanced level: Students will model total kitchen cooking energy profiles, perform CO₂ analysis, and develop evidence-based recommendations for reducing cooking-related energy use. |  |  |
| Time needed | 60 – 90 – 120 min |  |  |
| Resources | Basic level: Cooking appliance data table (in this worksheet); access to kitchen appliances at home or school. Medium level: Calculator; electricity tariff (€/kWh). Advanced level: Spreadsheet software; CO₂ emission factors; appliance specifications. |  |  |

# Basic level – Which cooking method uses the most electricity?

Introduction

We use electricity to cook food in many different ways: baking in an oven, heating in a microwave, boiling water in a kettle, or toasting bread. Each method uses a different amount of electricity depending on the power of the appliance and how long it runs.

In this section you will learn to identify common cooking appliances, compare their power ratings, and understand why some methods are more energy-efficient than others.

Theory

Every electrical cooking appliance converts electricity into heat. The key difference is HOW they do it:

An ELECTRIC OVEN heats a large enclosed space using heating elements. Because it heats a big volume of air, it uses a lot of energy (typically 2,000–2,500 W) and takes time to reach cooking temperature. Pre-heating alone can use 0.3–0.5 kWh.

A MICROWAVE OVEN uses electromagnetic waves to heat food directly, without heating the surrounding air. This makes it much more efficient for reheating and small portions (typically 600–1,200 W).

An ELECTRIC KETTLE is designed to boil water as quickly as possible using a powerful element (1,500–3,000 W). Although the power is high, the duration is short (2–5 minutes), so total energy per use is low.

A TOASTER uses radiant heating elements to brown bread. It typically runs at 800–1,500 W for 2–4 minutes per use.

An INDUCTION HOB uses a magnetic field to heat the pan directly, rather than a hot plate. It is approximately 85–90% efficient, compared to about 70% for a traditional ceramic hob.

Worked example Boiling 1.5 litres of water: • Electric kettle (2,200 W, 4 minutes): Energy = 2,200 × (4/60) ÷ 1,000 = 0.147 kWh • Electric oven (2,000 W, 15 minutes of preheating): Energy = 2,000 × (15/60) ÷ 1,000 = 0.500 kWh The kettle uses 3.4 times less energy than heating an oven for the same task!

Key terms

| Term | Definition |
| --- | --- |
| Power rating (W) | The rate at which an appliance uses electricity. Higher wattage = more energy per second. |
| Energy (kWh) | The total electricity consumed. Energy = Power × Time ÷ 1,000. |
| Efficiency | The percentage of electrical energy that is converted into useful heat for cooking. |
| Pre-heating | Running an oven empty to reach the target temperature before cooking begins. |
| Induction | A cooking technology that heats the pan directly using a magnetic field, rather than a hot surface. |

Student activity 1

Identifying cooking appliances and their power

Look at the cooking appliance data table below.

| Cooking appliance | Typical power (W) | Typical use time | Energy per use (kWh) |
| --- | --- | --- | --- |
| Electric oven | 2,200 W | 45 min | 1.65 |
| Microwave oven | 1,000 W | 5 min | 0.08 |
| Electric kettle | 2,200 W | 4 min | 0.15 |
| Toaster | 1,000 W | 3 min | 0.05 |
| Induction hob | 2,000 W | 20 min | 0.67 |
| Ceramic hob | 1,500 W | 25 min | 0.63 |

Which appliance has the highest power rating? Does the highest power always mean the highest energy consumption? Explain.

A student wants to reheat a plate of leftover pasta. Which appliance should they use – the oven or the microwave? Calculate the energy difference.

Why does the electric oven use so much more energy than the microwave, even though they can both heat food?

Rank all six appliances from most to least energy per typical use. Does anything surprise you?

Student activity 2

Matching the right appliance to the task

For each cooking task below, choose the most energy-efficient appliance and explain your choice.

Boiling water for tea (250 ml) → Which appliance? Why?

Reheating a bowl of soup → Which appliance? Why?

Baking a cake → Which appliance? Why? Can you suggest a way to reduce the energy used?

Making two slices of toast → Which appliance? Why?

# Medium level – Comparing cooking appliances: energy, time and cost

Introduction

Different cooking methods consume very different amounts of energy. At this level, you will calculate exact consumption figures, compare daily and annual costs, and evaluate whether changing cooking habits can reduce a household's electricity bill.

What you should already know: You should recognise common cooking appliances, understand power ratings, and know that Energy (kWh) = Power (W) × Time (h) ÷ 1,000.

Theory

A typical European household uses cooking appliances for 1–2 hours per day in total. The oven is usually the single biggest consumer because it runs for long periods at high power. Replacing oven use with microwave or induction hob where possible can reduce cooking energy by 30–60%.

Key formulas:
   Daily energy = Σ (Power_i × Time_i) ÷ 1,000 for each appliance i
   Monthly cost = Daily energy × 30 × Tariff
   Annual cost = Daily energy × 365 × Tariff

Worked example A family uses the oven once per day (2,200 W, 45 min) and the kettle three times per day (2,200 W, 4 min each). Oven: 2,200 × 0.75 ÷ 1,000 = 1.65 kWh/day Kettle: 3 × (2,200 × 4/60 ÷ 1,000) = 3 × 0.147 = 0.44 kWh/day Total: 2.09 kWh/day Monthly cost at €0.30/kWh: 2.09 × 30 × 0.30 = €18.81

Key terms

| Term | Definition |
| --- | --- |
| Daily energy profile | The total kWh consumed by all cooking appliances in a single day. |
| Tariff (€/kWh) | The price of electricity per kilowatt-hour. |
| Usage frequency | How many times per day or week an appliance is used. |
| Substitution | Replacing one cooking method with a more efficient alternative for the same task. |

Student activity 1

Daily cooking energy calculation

A household uses the following cooking appliances each day. Calculate the daily energy for each and the total. Use a tariff of €0.30/kWh.

| Appliance | Power (W) | Uses/day | Minutes/use | Daily kWh | Daily cost (€) |
| --- | --- | --- | --- | --- | --- |
| Electric oven | 2,200 | 1 | 45 |  |  |
| Microwave | 1,000 | 2 | 5 |  |  |
| Electric kettle | 2,200 | 4 | 4 |  |  |
| Toaster | 1,000 | 2 | 3 |  |  |
| Induction hob | 2,000 | 1 | 20 |  |  |
| Total |  |  |  |  |  |

Complete the table. Which single appliance contributes most to the daily energy consumption?

Calculate the monthly (30 days) and annual (365 days) cost for the total daily cooking energy.

Student activity 2

Scenario analysis: changing cooking habits

The family from Activity 1 decides to make two changes:
• Replace 3 out of 7 weekly oven uses with microwave (saving 40 minutes of oven time per replaced use, adding 8 minutes of microwave time)
• Boil only the water needed in the kettle, reducing boiling time from 4 min to 2.5 min per use

Calculate the new weekly energy consumption after these changes. How much energy (kWh) and money (€) does the family save per week?

Calculate the annual saving. Would you recommend these changes? Are there any disadvantages?

# Advanced level – Optimising kitchen energy use and carbon footprint

Introduction

Cooking accounts for approximately 10–15% of a household's total electricity consumption. Optimising cooking energy requires analysing the full picture: appliance efficiency, usage patterns, electricity tariffs, and carbon intensity of the grid. At this level you will build comprehensive models and develop evidence-based recommendations.

What you should already know: You should be able to calculate daily cooking energy profiles, compare appliance efficiencies, compute costs at different tariffs, and evaluate the impact of behaviour changes.

Theory

COOKING EFFICIENCY varies significantly between methods. An induction hob transfers approximately 85% of electrical energy to the food, a ceramic hob about 70%, and a gas hob about 40%. An oven loses energy through pre-heating, door openings, and heat escaping through the walls.

BATCH COOKING is a strategy where multiple meals are cooked at once, reducing the number of oven pre-heat cycles. This can save 30–40% of oven energy per week.

CO₂ IMPACT: The carbon footprint of cooking depends heavily on the electricity grid mix. In Norway (hydro-dominated, 0.02 kgCO₂/kWh), electric cooking is very clean. In Poland (coal-dominated, 0.71 kgCO₂/kWh), the same cooking produces 35× more CO₂.

Worked example A school cafeteria uses the following daily: Oven: 3,000 W × 3 h = 9.0 kWh Induction hobs: 4 × 2,000 W × 1.5 h = 12.0 kWh Kettles: 2 × 2,200 W × 0.5 h = 2.2 kWh Total: 23.2 kWh/day Annual CO₂ (Turkey, 0.44 kgCO₂/kWh): 23.2 × 200 school days × 0.44 = 2,042 kgCO₂ ≈ 2.0 tonnes CO₂/year

Student activity 1

School cafeteria energy audit

A school cafeteria operates 200 days per year. It uses the following equipment:

| Equipment | Power (W) | Hours/day | Daily kWh | Annual kWh | Annual cost (€0.30/kWh) |
| --- | --- | --- | --- | --- | --- |
| Large oven | 3,000 | 3.0 |  |  |  |
| Induction hobs (×4) | 2,000 each | 1.5 |  |  |  |
| Industrial kettle | 3,000 | 0.5 |  |  |  |
| Microwave (×2) | 1,000 each | 0.5 |  |  |  |
| Toaster (×2) | 1,200 each | 0.3 |  |  |  |
| Total |  |  |  |  |  |

Complete the table. What is the total annual energy consumption and cost?

Calculate the annual CO₂ emissions using emission factors for Germany (0.38), Turkey (0.44), and Poland (0.71). Express results in kg and tonnes.

Student activity 2

Efficiency upgrade analysis

The school considers two upgrades:
   A) Replace the oven with a more efficient model: 20% energy reduction, costs €2,500
   B) Replace ceramic hobs with induction: 25% energy reduction on hobs, costs €4,000

Calculate the annual energy and cost savings for each upgrade separately. Then calculate the payback period.

If the school can only afford one upgrade, which would you recommend? Consider both financial return and CO₂ reduction.

Student activity 3

National scale projection and policy recommendation

If 10,000 school cafeterias across Turkey each achieve a 20% reduction in cooking energy, calculate the total annual energy saving (kWh), cost saving (€), and CO₂ reduction (tonnes).

Write a 100–150 word policy recommendation to the Ministry of Education for improving cooking energy efficiency in school cafeterias. Include specific data from your calculations.