Worksheet 3 – Small kitchen appliances

| Student name |  | Date |  |
| --- | --- | --- | --- |
| Topic: | Basic level – Hidden energy users in the kitchen Medium level – Calculating the real cost of small appliances Advanced level – Standby power, phantom loads and smart solutions |  |  |
| Learning outcomes | Basic level: Students will identify small kitchen appliances (kettle, toaster, coffee machine), understand their power ratings, and recognise the concept of standby power. Medium level: Students will calculate consumption and cost for daily use patterns, compare appliances, and evaluate the impact of standby power. Advanced level: Students will model total small-appliance energy profiles, analyse phantom loads, evaluate smart plug solutions, and propose household efficiency strategies. |  |  |
| Time needed | 60 – 90 – 120 min |  |  |
| Resources | Basic level: Appliance data table (in this worksheet); observation of appliances at home. Medium level: Calculator; electricity tariff; standby power measurements or estimates. Advanced level: Spreadsheet software; smart plug specifications; CO₂ emission factors. |  |  |

# Basic level – Hidden energy users in the kitchen

Introduction

Besides the refrigerator and oven, kitchens contain many smaller electrical appliances: kettles, toasters, coffee machines, blenders, and more. Individually, each one uses relatively little energy. But used together, several times a day, they can add up to a surprising amount.

An often-overlooked issue is STANDBY POWER – the electricity consumed by appliances that are plugged in but not actively in use. A coffee machine displaying the time, a microwave with a glowing clock, or a kettle with an illuminated base all consume standby power continuously.

Theory

Small kitchen appliances generally have high power ratings but short usage times. An electric kettle uses 2,000–3,000 W but only runs for a few minutes. The total energy per use is therefore small.

STANDBY POWER typically ranges from 0.5 to 5 W per appliance. This seems negligible, but a device consuming 3 W continuously uses:
   3 W × 24 h × 365 days ÷ 1,000 = 26.3 kWh per year
With 5–10 appliances on standby in a kitchen, this can exceed 100–200 kWh per year – equivalent to running a modern refrigerator.

Worked example A coffee machine uses 1,200 W during brewing (5 min) and 3 W on standby (23 h 55 min). Active: 1,200 × 5/60 ÷ 1,000 = 0.10 kWh Standby: 3 × 23.92/1,000 = 0.072 kWh Total daily: 0.172 kWh Surprisingly, standby accounts for 42% of the daily consumption!

Key terms

| Term | Definition |
| --- | --- |
| Standby power | Electricity consumed by a plugged-in appliance when it is not performing its main function. |
| Phantom load | Another name for standby power – energy "leaking" from appliances that appear to be off. |
| Smart plug | A device that can automatically cut power to appliances, eliminating standby consumption. |
| Power strip | A multi-socket extension with an on/off switch, allowing multiple appliances to be truly disconnected. |
| Duty cycle | The fraction of time an appliance is actively operating versus idle or on standby. |

Student activity 1

Small appliance inventory

Look at the table below showing typical small kitchen appliances.

| Appliance | Active power (W) | Typical use per day | Standby power (W) |
| --- | --- | --- | --- |
| Electric kettle | 2,200 W | 4 min × 3 uses = 12 min | 0.5 W |
| Toaster | 1,000 W | 3 min × 2 uses = 6 min | 0 W (no standby) |
| Coffee machine (drip) | 1,200 W | 5 min × 2 uses = 10 min | 3 W |
| Coffee machine (capsule) | 1,400 W | 1 min × 3 uses = 3 min | 1 W |
| Blender | 500 W | 2 min × 1 use = 2 min | 0 W |
| Electric hand mixer | 300 W | 5 min × 0.5 use = 2.5 min | 0 W |

Which appliance has the highest power rating? Which has the lowest? Does higher power mean higher daily energy use?

Which appliances have standby power? How many hours per day are they on standby?

A student says: "My kettle uses 2,200 W, so it must be the biggest energy waster in the kitchen." Do you agree or disagree? Explain.

Name two actions you can take today to reduce standby power consumption in your kitchen.

Student activity 2

Standby power detectives

Walk around your kitchen at home (or the school kitchen). List every appliance that is plugged in and identify which ones have a light, display, or indicator when not in active use.

List at least 5 kitchen appliances that are plugged in at home. For each, note whether it has a visible standby indicator (light, clock, display).

Estimate the total standby power in your kitchen (in Watts). Multiply by 24 hours and 365 days to estimate the annual standby energy in kWh.

# Medium level – Calculating the real cost of small appliances

Introduction

At this level you will calculate the exact energy consumption and cost of small kitchen appliances, separating active use from standby power. You will discover that standby consumption can rival or exceed active consumption for some appliances.

What you should already know: You should be able to identify small kitchen appliances and their power ratings, understand the concept of standby power, and know the formula Energy (kWh) = Power (W) × Time (h) ÷ 1,000.

Theory

For appliances with standby power, the total daily energy consumption has two parts:
   Total daily kWh = Active kWh + Standby kWh
   Active kWh = Active power (W) × Active time (h) ÷ 1,000
   Standby kWh = Standby power (W) × Standby time (h) ÷ 1,000
   Standby time = 24 h – Active time

Worked example Electric kettle: 2,200 W active, 12 min/day active, 0.5 W standby. Active: 2,200 × (12/60) ÷ 1,000 = 0.440 kWh Standby: 0.5 × (23.8) ÷ 1,000 = 0.012 kWh Total: 0.452 kWh/day → 165 kWh/year Annual cost at €0.30/kWh: €49.50 Standby share: 0.012/0.452 = 2.7% Coffee machine: 1,200 W active, 10 min/day, 3 W standby. Active: 1,200 × (10/60) ÷ 1,000 = 0.200 kWh Standby: 3 × (23.83) ÷ 1,000 = 0.071 kWh Total: 0.271 kWh/day → 99 kWh/year Standby share: 0.071/0.271 = 26.4%

Key terms

| Term | Definition |
| --- | --- |
| Active consumption | Energy used while the appliance is performing its main function. |
| Standby consumption | Energy used while the appliance is plugged in but idle. |
| Standby ratio | The percentage of total energy that comes from standby power. |
| Smart power strip | A power strip that automatically cuts standby power when appliances are not in use. |

Student activity 1

Complete energy profile

Calculate the daily energy for each appliance, separating active and standby. Use a tariff of €0.30/kWh.

| Appliance | Active kWh/day | Standby kWh/day | Total kWh/day | Annual kWh | Annual cost (€) | Standby % |
| --- | --- | --- | --- | --- | --- | --- |
| Electric kettle |  |  |  |  |  |  |
| Toaster |  |  |  |  |  |  |
| Coffee machine (drip) |  |  |  |  |  |  |
| Coffee machine (capsule) |  |  |  |  |  |  |
| Blender |  |  |  |  |  |  |
| Total |  |  |  |  |  |  |

1-Complete the table using the appliance data from the Basic level. Which appliance has the highest standby percentage?

2-If the family eliminates ALL standby power by unplugging appliances or using a switched power strip, how much energy and money would they save per year?

Student activity 2

Kettle efficiency experiment

The amount of energy a kettle uses depends on how much water you boil. Many people boil more water than they need.

1-Calculate the energy used to boil 250 ml (1 cup), 500 ml (2 cups), 1 litre, and 1.5 litres. Assume 2,200 W and boiling times of 1.5 min, 2.5 min, 4 min, and 5.5 min respectively.

2-If someone always boils 1.5 litres but only uses 250 ml (one cup), how much energy is wasted per use? How much per year (assuming 3 uses/day)?

# Advanced level – Standby power, phantom loads and smart solutions

Introduction

Phantom loads from small appliances represent one of the easiest areas for energy savings in a household. Smart plugs and timed power strips can automatically eliminate standby power. At this level you will model the total phantom load of a kitchen, evaluate smart plug solutions, and analyse the broader environmental impact.

What you should already know: You should be able to calculate active and standby energy separately, compute standby percentages, and understand the concept of phantom loads.

Theory

A SMART PLUG is a device inserted between the wall socket and the appliance. It can be programmed to cut power completely at certain times, or to switch off automatically after a period of inactivity. Smart plugs typically cost €10–25 each.

The PAYBACK PERIOD for a smart plug depends on:
   Payback (years) = Smart plug cost (€) ÷ Annual standby cost saved (€/year)
For appliances with high standby power (3–5 W), payback can be less than one year.

Worked example A smart plug costs €15. It eliminates 3 W of standby from a coffee machine. Annual standby eliminated: 3 × 24 × 365 ÷ 1,000 = 26.3 kWh Annual saving: 26.3 × 0.30 = €7.89 Payback: 15 ÷ 7.89 = 1.9 years

Student activity 1

Smart plug payback analysis

A family considers buying smart plugs (€15 each) for their kitchen appliances. Evaluate whether each appliance justifies a smart plug.

| Appliance | Standby (W) | Annual standby kWh | Annual standby cost (€) | Payback (years) | Worth it? |
| --- | --- | --- | --- | --- | --- |
| Coffee machine | 3.0 |  |  |  |  |
| Microwave | 2.0 |  |  |  |  |
| Capsule machine | 1.0 |  |  |  |  |
| Kettle | 0.5 |  |  |  |  |
| Oven display | 4.0 |  |  |  |  |

1-Complete the table. Which appliances have a payback of less than 2 years? Which ones are not worth a smart plug?

2-If the family buys smart plugs for all appliances with a payback under 3 years, what is the total investment, total annual saving, and combined payback period?

Student activity 2

National phantom load impact

1-The average Turkish household has 8 kitchen appliances with an average standby power of 2 W each. There are approximately 25 million households in Turkey. Calculate the total national standby energy waste in GWh/year and the CO₂ emissions (Turkey: 0.44 kgCO₂/kWh).

2-Write a 100–150 word recommendation for a public awareness campaign about kitchen standby power. Include at least three specific actions households can take, supported by numbers from your calculations.